# EE 316 Computer Engineering Junior Lab Design Project 3

Spring 2025

Specification: Signal Generation using sensors with I2C interface Due Date:	Thursday, February 27 - lab demo

Tuesday, March 1, written report due.

Parts list:

- Trenz ZynqBerry board with GPIO Expansion board

- One function generator

- One 16x2 character I2C LCD display module

- A PCF8591 ADC-DAC module with I2C interface

- A couple of Op-AMPs (LM741/LM358 compatible), capacitors, resistors, etc.

Design a system to sample, store, display, and (re)create analog waveforms. Reset uses PMOD button BTN0. In this project, use Vivado 2019.1 with the board files (already installed on AppsAnywhere).

Op-Amp adder circuit: Periodic sinusoidal waveforms will be generated by function generators. The input waveform should have a peak-to-peak amplitude of 3.3 Volts. Note: The ADC should sample positive analog voltages; thus, the input signal must be added to 3.3 volts using an op-amp adder with a gain of 1/2.

ADC: The PCF8591 module has four input channels: AIN0, AIN1, AIN2, and AIN3. The output of the Op-Amp Circuit (ANALOG_INPUT) should be connected to AIN2. The other three Inputs can be connected to three sensors using three jumpers, as shown below:

- AIN0 – Jumper P5 – Light Dependent Resistor (LDR)

- AIN1 – Jumper P4 – Thermistor (TEMP)

- AIN3 – Jumper P6 – Potentiometer (POT)

PWM: The system should have a PWM output. The duty cycle of the PWM signal will be proportional to the 8-bit value of the data. The user can select between 4 inputs to determine the PWM duty cycle using the PMOD button (BTN1). The PWM output should pass through a low-pass filter to reconstruct the input signals displayed on the Oscilloscope. For AIN2 input, a sinusoidal waveform will be displayed on the oscilloscope along with the original input. What is the highest input frequency the system can reconstruct in your design?

Clock Generation: The system will also be able to generate a clock output. The frequency of the clock will vary between 500 and 1500 Hz and is controlled by the Potentiometer, the Thermistor, and the LDR. The clock generation module can be enabled or disabled using BTN2. You can use PMOD LEDs to show the states.

LCD: The Zynq board will have an I2C LCD connected. Use the 4-bit data option for the LCD. The LCD will display the selected source (LDR, TEMP, or POT) that generates the PWM signal on the first line. The second line of the LCD display, “Clock Output,” should indicate if the system is currently generating a clock.

# Other objectives are:

- to maximize the bandwidth of analog signals that can be sampled, stored, and analyzed without aliasing

- to make sure the error for the desired frequency is minimized.

# Useful Links:

- P CF8591 – 8-bit ADC and DAC converter

- A rduino PCF8591 Digital to Analog Tutorial

- L esson 12 PCF8591 AD Converter

# Teams:

| Team 1 | Team 2 | Team 3 | Team 4 | Writer |
| --- | --- | --- | --- | --- |
| Adamson | Isaak | Afolayanka | Uon |  |
| Zalepeski | Scott | Elkins | Lamontagne |  |
|  | Carroll | Nevin - D'Allaird | Campo |  |